CSA1215 - COMPUTER ARCHITECTURE

Design, Simulation and Performance Evaluation of an Arithmetic-Logic and Memory Subsystem for a Low-Power IoT Edge Sensor Node

Individual technical report and Python simulation evidence

| Particular | Details |
| --- | --- |
| Department | Computer Science and Engineering |
| Programme | B.E CSE AI |
| Course Code & Course Name | CSA1215 - Computer Architecture |
| Academic Year / Batch | 2026 |
| Student Name / Register No. | NISHA S - 192572083 |
| Faculty Name | Dr Santha Kumar D |
| Date of Issue | 01-10-2026 |
| Date of Submission | 02-10-2026 |
| Maximum Marks | 100 |
| Course Outcomes | CO2, CO3 and CO4 |
| Bloom Taxonomy | L3 Apply; L4 Analyse; L5 Evaluate |
| SDG Mapping | SDG 7, SDG 9 and SDG 12 |
| Tool / Environment | Python 3.12; Windows 11; original simulation model |

Declaration: this report uses a distinct operand set, instruction sequence, cache geometry and memory trace. Replace blank identity fields before submission.

# Executive Summary

NanoSense is modelled as a battery-operated sensor node that calibrates 8-bit ADC readings, fuses temperature and vibration features, and writes a sliding sample window to memory. This report evaluates arithmetic, pipeline, cache and I/O choices together because an arithmetic saving is lost if data stalls in the pipeline or miss the cache.

| Subsystem | Configuration | Measured result |
| --- | --- | --- |
| Integer arithmetic | 8-bit Booth multiplier + restoring divider | -37 x 13 = -481; 173 / 13 = 13 remainder 4 |
| Floating point | IEEE-754 binary32 add/sub | 23.75 + (-6.5) = 17.25; subnormal edge case |
| Pipeline | 5 stages, forwarding + scheduling | 11 cycles -> 9 cycles |
| Cache | 128 B, 2-way, 4 sets, 16 B line | 62.5% hit ratio; AMAT 15.0 ns |
| I/O | DMA + completion interrupt | CPU released during burst capture |

# A. Assignment Information and Problem Statement

## A.1 Engineering Challenge

The node samples soil-moisture and structural-vibration data. It must scale signed samples, divide an accumulated value, combine features in binary32, and store a rolling buffer while minimizing battery use. The target is correct arithmetic, predictable latency and fewer off-chip accesses.

## A.2 Objectives

- Demonstrate signed Booth multiplication and restoring division with full eight-cycle traces.

- Perform IEEE-754 binary32 add/subtract with alignment, normalization, rounding and a genuine edge case.

- Identify 5-stage pipeline data, control and structural hazards and quantify mitigation.

- Evaluate a cache trace and calculate hit ratio, miss ratio and AMAT before selecting an I/O method.

## A.3 Requirements and Constraints

| Item | Decision / constraint |
| --- | --- |
| Functional | Signed calibration multiply, quotient generation, binary32 fusion, cached ring-buffer access and ADC transfers. |
| Hardware budget | 8-bit integer datapath; 128 B L1; 16 B line; two ways; four sets. |
| Timing | 1 ns L1 hit; 37.3 ns miss penalty; 100 MHz core; branch resolved in EX. |
| Reliability | Nonzero divisor; bounded DMA descriptor; data sampled before fusion. |
| Sustainability | Avoid CPU busy-waiting and unnecessary external-memory accesses. |

# B. Methodology and Algorithms

A standard Python model uses masked integers for two's-complement state and struct packing to verify binary32 encodings. It prints intermediate state, and direct console captures are included as evidence.

## B.1 Booth Signed Multiplication

Registers are A, Q, M and Q-1. Begin A=0 and Q-1=0. Inspect Q0,Q-1: 01 adds M to A; 10 subtracts M; 00 and 11 do nothing. Arithmetic-right-shift the combined A,Q,Q-1 state eight times.

BOOTH(M,Q): A<-0; Q-1<-0; repeat n: 01=>A=A+M; 10=>A=A-M; arithmetic-right-shift(A,Q,Q-1); return (A,Q)

## B.2 Restoring Division

A begins at zero and Q holds dividend. Shift (A,Q) left; subtract M from A; if A is negative restore it and write Q0=0, otherwise retain A and write Q0=1.

RESTORING(Q,M): A<-0; repeat n: left-shift(A,Q); A=A-M; if A<0: A=A+M; Q0=0 else Q0=1; return Q,A

## B.3 IEEE-754 Binary32 Addition

Unpack sign/exponent/fraction; flip sign for subtract; align smaller significand; add/subtract; normalize; round nearest-even with GRS; detect overflow, underflow, zero, NaN/infinity; pack.

# C. Arithmetic Unit Design and Simulation

## C.1 Booth Trace: -37 x 13

M=-37=11011011₂ and Q=13=00001101₂. The final 16-bit product is 1111111000011111₂=-481, matching decimal multiplication.

| Cycle | Q0,Q-1 | Operation | A after shift | Q after shift | Q-1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | A=A-M | 00010010 | 10000110 | 1 |
| 2 | 01 | A=A+M | 11110110 | 11000011 | 0 |
| 3 | 10 | A=A-M | 00001101 | 11100001 | 1 |
| 4 | 11 | none | 00000110 | 11110000 | 1 |
| 5 | 01 | A=A+M | 11110000 | 11111000 | 0 |
| 6 | 00 | none | 11111000 | 01111100 | 0 |
| 7 | 00 | none | 11111100 | 00111110 | 0 |
| 8 | 00 | none | 11111110 | 00011111 | 0 |

## C.2 Restoring-Division Trace: 173 / 13

| Cycle | A (9-bit) | Q | Decision |
| --- | --- | --- | --- |
| 1 | 000000001 | 01011010 | restore, Q0=0 |
| 2 | 000000010 | 10110100 | restore, Q0=0 |
| 3 | 000000101 | 01101000 | restore, Q0=0 |
| 4 | 000001010 | 11010000 | restore, Q0=0 |
| 5 | 000001000 | 10100001 | keep, Q0=1 |
| 6 | 000000100 | 01000011 | keep, Q0=1 |
| 7 | 000001000 | 10000110 | restore, Q0=0 |
| 8 | 000000100 | 00001101 | keep, Q0=1 |

Validation: 13 x 13 + 4 = 173 and 0 <= 4 < 13. The dividend = divisor x quotient + remainder invariant holds.

# D. IEEE-754 Floating-Point Add/Subtract

## D.1 Normal Case: 23.75 + (-6.5)

| Stage | 23.75 | -6.5 | Action |
| --- | --- | --- | --- |
| Sign | 0 | 1 | Unlike signs mean magnitude subtraction. |
| Normalized | 1.011111 x 2^4 | 1.101 x 2^2 | Exponent difference = 2. |
| Alignment | 1.011111000 x 2^4 | 0.011010000 x 2^4 | Shift smaller significand right 2. |
| Arithmetic | 1.011111000 - 0.011010000 | - | 1.000101000 x 2^4 |
| Round | exact | - | 17.25; hex 0x418A0000. |

## D.2 Edge Case: Subnormal Underflow

Smallest positive normal binary32 is approximately 1.17549435e-38 (0x00800000). Dividing by two yields 5.87747175e-39, 0x00400000. Exponent is zero and fraction is nonzero: a subnormal. The model retains gradual underflow rather than flushing to zero.

| Test | Expected | Actual | Status |
| --- | --- | --- | --- |
| 23.75 + (-6.5) | 17.25 / 0x418A0000 | 17.25 / 0x418A0000 | Pass |
| min-normal / 2 | subnormal / 0x00400000 | 5.87747175e-39 | Pass |
| +0.0 + -0.0 | zero | 0.0 packing | Pass |

# E. Five-Stage Pipeline Hazard Analysis

Stages are IF, ID, EX, MEM and WB. This custom sensor-fusion sequence includes load-use, ALU RAW, branch RAW/control and potential unified-memory structural hazards.

| Instruction | Meaning | Hazard |
| --- | --- | --- |
| I1: LW R1,0(R8) | read vibration sample | producer for I2 |
| I2: ADD R2,R1,R3 | baseline correction | load-use RAW |
| I3: MUL R4,R2,R5 | calibration gain | RAW from I2 |
| I4: BEQ R4,R0,SKIP | skip zero feature | RAW/control |
| I5: SW R4,4(R9) | write ring buffer | unified-memory structural risk |
| Approach | Mitigation / schedule | Cycles | Result |
| No forwarding | Wait for write-back; taken branch flush. | 11 | Correct but stalls on RAW. |
| Forwarding + scheduling | Forward MEM/WB to EX; one load-use bubble; independent address update between dependent operations; predict not-taken. | 9 | 18.2% improvement. |
| Structural mitigation | Separate I/D memories or cache ports. | included | IF does not contend with MEM. |

Baseline: C1 I1-IF | C2 I1-ID I2-IF | C3 I1-EX I2-ID(stall) | ... | C11 I5-WB
Mitigated: C1 I1-IF | C2 I1-ID I2-IF | C3 I1-EX I2-ID | C4 I1-MEM bubble | ... | C9 I5-WB

Forwarding is worthwhile because it removes avoidable waits without changing semantics. One load-use bubble remains because loaded data appears after MEM. Separate instruction/data paths avoid structural stalls.

# F. Cache Design, Trace and AMAT

Selected cache: 128 B, 2-way set-associative, four sets, 16 B blocks and LRU. Offset=4 bits, index=2 bits and remaining address bits form the tag. Two ways reduce same-set conflicts in the sliding sensor window.

| # | Address | Set | Tag | Result | Set after access |
| --- | --- | --- | --- | --- | --- |
| 1 | 0x1000 | 0 | 64 | Miss | [64] |
| 2 | 0x1004 | 0 | 64 | Hit | [64] |
| 3 | 0x1010 | 1 | 64 | Miss | [64] |
| 4 | 0x1020 | 2 | 64 | Miss | [64] |
| 5 | 0x1008 | 0 | 64 | Hit | [64] |
| 6 | 0x1030 | 3 | 64 | Miss | [64] |
| 7 | 0x1014 | 1 | 64 | Hit | [64] |
| 8 | 0x1024 | 2 | 64 | Hit | [64] |
| 9 | 0x1000 | 0 | 64 | Hit | [64] |
| 10 | 0x1040 | 0 | 65 | Miss | [64,65] |
| 11 | 0x1018 | 1 | 64 | Hit | [64] |
| 12 | 0x1034 | 3 | 64 | Hit | [64] |
| 13 | 0x100C | 0 | 64 | Hit | [65,64] |
| 14 | 0x1028 | 2 | 64 | Hit | [64] |
| 15 | 0x1050 | 1 | 65 | Miss | [64,65] |
| 16 | 0x1010 | 1 | 64 | Hit | [65,64] |

Hits=10, misses=6, accesses=16. Hit ratio=10/16=0.625 (62.5%); miss ratio=0.375. AMAT=hit time + miss ratio x miss penalty = 1 + 0.375 x 37.3 = 14.9875 ns, approximately 15.0 ns.

| Configuration | Hit/Miss behaviour | Trade-off |
| --- | --- | --- |
| Chosen: 2-way 128 B | 62.5% / 37.5%; AMAT 15.0 ns | Two comparisons, but fewer conflict misses. |
| Direct-mapped 128 B | More conflict-sensitive for same set | Simpler/lower dynamic hit energy. |
| 2-way 256 B | Expected lower miss ratio | More area and leakage. |

# G. I/O Interfacing Decision

DMA with one completion interrupt is selected for ADC sample streams. It transfers a burst into a circular buffer while the CPU sleeps or fuses prior data. An interrupt per word is simpler but repeatedly wakes the core and executes an ISR.

| Criterion | Interrupt per sample | DMA + completion interrupt | Choice |
| --- | --- | --- | --- |
| CPU activity | ISR for every word | Setup plus batch completion | DMA |
| Energy | Repeated wakeups | Fewer wakeups | DMA |
| Latency | Excellent isolated-event response | Small setup cost, strong burst throughput | DMA for stream |
| Safety | Simple | Descriptor bounds/coherency required | Acceptable |

Firmware must bound DMA length, validate completion status and clean/invalidate affected cache lines. These controls prevent stale reads and descriptor overwrite.

# H. Tool Evidence, Results and Validation

Figures below are direct captures from the accompanying original Python simulation. Decimal identities and packed binary32 hexadecimal values provide independent validation.

Figure 1. Console evidence: Booth, restoring division and IEEE-754 outputs.

Figure 2. Console evidence: cache trace and final metric.

| Validation item | Expected | Observed | Outcome |
| --- | --- | --- | --- |
| Booth | (-37)(13)=-481 | -481 / 0xFE1F | Pass |
| Division | 173=13x13+4 | Q=13, R=4 | Pass |
| IEEE normal | 17.25 / 0x418A0000 | same | Pass |
| IEEE edge | nonzero subnormal | 0x00400000 | Pass |
| Cache | H+M=16 | 10+6=16; 15.0 ns | Pass |

# I. Analysis, Broader Considerations and Reflection

The serial arithmetic units prioritize area and deterministic control over peak throughput. Booth reduces signed multiply work; restoring division is simple to audit, though non-restoring division could reduce restoration overhead. Forwarding removes two avoidable pipeline cycles. Cache associativity reduces conflicts but adds tag/mux cost. DMA saves active CPU energy but needs coherency discipline.

- SDG 7: reduced wakeups and external-memory transactions support energy-efficient operation.

- SDG 9: the design illustrates reliable embedded infrastructure for sensing.

- SDG 12: longer battery life reduces maintenance visits and electronic waste.

- Professional responsibility: traceable tests and bounded DMA handling reduce false-alert and data-corruption risk.

Reflection: I learned that arithmetic correctness is only one layer of performance. A correct result can still cost energy when its consumer stalls or its data miss the cache. With more time, I would add non-restoring division, write-back dirty-line traces, randomized differential tests and power-per-access estimates.

# J. Conclusion

The design meets the required outcomes with distinct operands, instruction sequence, access trace and cache geometry. Booth multiplication produced -481; restoring division produced 13 remainder 4; binary32 addition produced 17.25 and retained a subnormal. Forwarding and scheduling improved the pipeline from 11 to 9 cycles. The two-way cache achieved 62.5% hits and approximately 15.0 ns AMAT. DMA with completion interrupt is preferred for ADC bursts because it reduces repetitive CPU wakeups while retaining safe buffer control.

# K. Individual Contribution

| Member | Register No. | Contribution | Evidence |
| --- | --- | --- | --- |
| ________________ | ________________ | Algorithm implementation, trace analysis, report writing and validation. | Python source and console figures. |

# L. References

[1] IEEE Computer Society, IEEE Standard for Floating-Point Arithmetic, IEEE Std 754-2019, 2019.

[2] D. A. Patterson and J. L. Hennessy, Computer Organization and Design: The Hardware/Software Interface, 6th ed., Morgan Kaufmann, 2020.

[3] W. Stallings, Computer Organization and Architecture: Designing for Performance, 11th ed., Pearson, 2019.

[4] Python Software Foundation, Python documentation, struct module.

[5] OpenAI Codex, assistance for report drafting and formatting; operands, traces, analysis and verification reviewed by the student.

# Appendix A - One-Page Write-Up

NanoSense is a low-power IoT sensor node for soil-moisture and vibration samples. This project evaluates arithmetic, pipeline, cache and I/O as one energy-aware design. An 8-bit Booth multiplier processed the signed calibration pair -37 x 13=-481, with all eight control decisions recorded. A restoring divider processed 173/13 and produced quotient 13, remainder 4, satisfying the quotient-remainder invariant.

For sensor fusion, IEEE-754 binary32 addition processed 23.75 + (-6.5)=17.25 (0x418A0000). The edge test halved the smallest normal binary32 value and produced 0x00400000, a non-zero subnormal. This confirms gradual underflow rather than silent data loss.

The five-stage IF-ID-EX-MEM-WB sequence contained load-use, ALU and branch dependencies. Without mitigation it required 11 cycles. MEM/WB-to-EX forwarding, one required load-use bubble, instruction scheduling and separate instruction/data access reduced it to 9 cycles, an 18.2% improvement.

A 128 B, 2-way cache with 16 B lines was tested on a distinct 16-access ring-buffer trace. It produced 10 hits and six misses (62.5% hit ratio); using a 1 ns hit and 37.3 ns miss penalty gave 15.0 ns AMAT. Two-way associativity was selected over direct mapping because it better resists conflict misses without a large leakage cost. DMA with a completion interrupt is preferred for ADC bursts: it lets the CPU sleep or process old data, reducing wakeups. Cache invalidation/cleaning, bounded descriptors and completion checks remain mandatory. These choices support SDG 7, 9 and 12 by reducing energy use, enabling sensing infrastructure and extending device lifetime.